Example 2 – A medium-height office building in a tropical city (NEW MODEL)

Design wind loads are required for a proposed office building.

The relevant information is as follows :

- Location : A tropical city in a typhoon/cyclone zone

- Terrain : Suburban terrain for all directions

- Topography : flat (ground slope less than 1 in 20) for greater than 5km in all directions.

- Design wind speed at 48m (all-direction):

3-second gust   56 m/s

  - 10-minute mean  36 m/s

  - 1-hour mean    33 m/s

- Dimensions :  average roof height : H = 48m

Horizontal dimensions: 30m × 60m (rectangular cross-section).

- Reinforced concrete frame construction.

- The building is air-conditioned with non-opening windows, and can be considered effectively sealed.

- Mullions for glazing panels are spaced at 1.5m.

- Roof is flat with a 0.9m high parapet.

- Sway frequency  f1 = 1.2 Hz

- Damping ratio  1 = 0.02

- Turbulence intensity at 48m  Iu,48m = 0.2

Please calculate using your code/standard :

1) The along-wind base bending moment (MNm) and base shear (kN) for wind directions normal to the 60m wall

2) Cladding pressures (kPa) on window elements 4.5m2  in area, within 3m of corners, for the top level (45 to 48m above ground level) on all walls.

Worked Example 2  NEW MODEL

| Economy | Basic wind speed U0 (m/s) | Frequency fsway (Hz) | Damping sway | Power-law index  | Gradient height ZG(m) | Design wind speed at top UH (m/s) | Design velocity pressure at top qH (N/m2) | Tip Turbulence intensity IH | Tip Turbulence scale LH (m) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| AS/NZS | 3s-gust 46 44 | 1.2 | 0.02 |  |  | Overall load 56 Cladding 56 |  | 0.2 |  |
| China | 10min-mean 32 |  |  | 0.22 |  | 36 |  |  |  |
| Hong Kong | 3s-gust 50 |  |  | 0.11 | 500 | 56 |  |  | 1480 |
| India | 3s-gust 45 |  |  |  |  | 56 |  |  |  |
| Indonesia | AS/NZS 3s-gust |  |  |  |  | 56 |  |  |  |
| Japan | 10min-mean 33 |  |  | 0.20 | 450 | 36 | 790 |  | 126 |
| Korea | 10min-mean 34 |  |  | 0.22 | 400 | 36 | 795 |  | 126 |
| Malaysia | 3s-gust 52 |  |  |  |  | 56 |  |  |  |
| Philippines | 3s-gust 53 |  |  |  |  | 56 |  |  |  |
| Singapore | 3s-gust 45 |  |  |  |  | 56 | 1,882 |  |  |
| Taiwan | 10min-mean 37 |  |  | 0.25 | 400 | 36 | 761 |  | 139 |
| Thailand | 1h-mean 33.5 |  |  | 0.25 | 317.5 | 33 | 681 |  | 1,220 |
| Vietnam | 3s-gust 42.3 |  |  | 0.18 | 400 | 56 |  |  |  |
| ASCE | 3s-gust 49.5 |  |  | 0.14, 0.25 | 366 |  | 1,440 |  | 165 |
| NBCC | 1h-mean 33 |  |  | 0.3 |  | 33 |  |  |  |
| EUROCODE | 10min-mean 32.9 |  |  |  |  | 36 | 1,940 |  |  |

Worked Example 2  NEW MODEL

| Economy | Dynamic response factor Cdyn Gust loading factor GD | Windward Cp for overall load Cpe1 | Leeward Cp for overall load Cpe2 | Net peak cladding force coefficient Ĉc+ | Net peak cladding force coefficient Ĉc- | Equivalent internal pressure coefficient Cpi* | Base Shear Q (kN) | Base Bending Moment M (MNm) | Cladding Pressure P+ (kPa) | Cladding Pressure P (kPa) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| AS/NZ | Cdyn 1.0 | 0.8 varying with z | 0.5 lee 0.65 side | 1.2 | 1.95 | 0.2 and 0 | 5,727 | 150 | 2.25 | 3.67 |
| China | GD 1.0 | 0.8 | 0.5 | 1.0 (GD 1.73) | 2.0 reduction (0.87) |  | 3,282 | 99 | 1.22 | 2.44 |
| Hong Kong | n/a | n/a | n/a | 1.0 | 1.4 | n/a | 4,573 | 116 | 1.87 | 2.62 |
| India |  | 0.7 | 0.4 lee 0.5 side |  | 1.2 | 0.3 and 0.2 | 4,957 | 131 | 1.55 | 2.26 |
| Indonesia | 1.0 | 0.8 | 0.5. 0.3 | ? | ? | ? | 5,975 | 169 | 2.24 | 3.64 |
| Japan | GD 2.04 | (0.17Z2) 0.732 | 0.5 | 2.71 | 3 | 0.5 and 0 | 5,061 | 132 | 2.14 | 2.37 |
| Korea | GD 2.2 | 0.8 | 0.5 | 1.92 | 3.2 | 0.52 and 0 | 5,534 | 134 | 1.53 | 2.54 |
| Malaysia | Cdyn 1.0 | 0.8 varying with z | 0.5 lee 0.65 side | 1.22 | 1.95 | 0.2 and 0 | 5,698 | 152 | 2.26 | 3.70 |
| Philippines | Cdyn 0.8 | 0.8 | 0.5 | 0.8 | 1.6 | 0.18 | 5,026 | 128 | 1.32 | 2.85 |
| Singapore |  | 0.8 | 0.5 | 1.0(0.81.25) | 1.95(0.653) | 0.2 and 0 | 6,556 | 163 | 2.26 | 3.67 |
| Taiwan | GD 1.73 | 0.8 | 0.5 | GCp only 1.7 parapet 1.8 | GCp only 3.5 parapet 1.1 | GCpi 0.375 | 3,738 | 99.5 | 1.58 | 2.95 |
| Thailand | 2.06 | 0.8 | 0.5 | +2.25 | -3 | 0.15 and 0 | 3,737 | 97.1 | 1.86 | 2.23 |
| Vietnam | n/a | 0,8 | 0.6 lee 0.6 side | parapet 1.4 | n/a | n/a | 6,423 | 165 | 2.44 | 1.83 |
| ASCE | 0.825 | 0.8 | 0.5 | 0.8 | 1.6 | 0.18 and 0.18 | 4,108 | 117 | 1.41 | 2.56 |
| NBCC | 2.0 | 0.8 | 0.5 | 2.50.9 | 1.22.5 | 0.152.0 | 5,332 | 142 | 1.80 | 2.11 |
| EUROCODE | 1.0 | 0.8 | 0.53 | 5.57 | 1.27 | 0.3 and 0.2 | 6,042 | 182 | 1.69 | 2.47 |

Worked Example 2  ORIGINAL MODEL

| Economy | Basic wind speed U0 (m/s) | Frequency fsway (Hz) | Damping sway | Power-law index  | Gradient height ZG(m) | Design wind speed at top UH (m/s) | Design velocity pressure at top qH (N/m2) | Turbulence intensity at the top IH | Turbulence scale at the top LH (m) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| AS/NZS | 3s-gust 52 |  |  |  |  | Overall load 63.2 Cladding 66.6 |  | n/a |  |
| China | 10min-mean 35 |  |  | 0.22 |  | 38.8 |  | 0.100 |  |
| Hong Kong | 3s-gust 52 | 1.2 1.4 | n/a | 0.11 | 500 | 58.6 | 2.06 | 0.113 | 1480 |
| India | 3s-gust 52 |  |  |  |  | 55.7 |  | n/a |  |
| Indonesia |  |  |  |  |  |  |  |  |  |
| Japan | 10min-mean 35 | 1.2 | 0.02 | 0.20 | 450 | 38.0 | 882 | 0.175 | 126 |
| Korea | 10min-mean 35 | - | - | 0.22 | 400 | 37.0 | 851 | 0.143 |  |
| Malaysia |  |  |  |  |  |  |  |  |  |
| Philippines | 3s-gust 52 |  |  |  |  | 55.0 |  | n/a |  |
| Singapore | 3s-gust 52 |  |  |  |  | 64.9 |  | n/a |  |
| Taiwan | 10min-mean 35 | 1.2 | 0.02 | 0.25 | 400 | 34.5 | 693 | 0.250 | 139 |
| Thailand | 1h-mean 31 |  | 0.02 |  |  | 37.0 | 856 | n/a |  |
| Vietnam | 3s-gust 52 | 1.2 | n/a | 0.18 | 400 | 69.2 |  |  |  |

Worked Example 2  ORIGINAL MODEL

| Economy | Dynamic response factor Cdyn Gust loading factor GD | Windward Cp for overall load Cpe1 | Leeward Cp for overall load Cpe2 | Net peak cladding force coefficient Ĉc+ | Net peak cladding force coefficient Ĉc- | Equivalent internal pressure coefficient Cpi* | Base Shear (kN) | Base Bending Moment (MNm) | Cladding Pressure + (kPa) | Cladding Pressure  (kPa) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| AS/NZ | Cdyn 1.0 | 0.8 varying with z | 0.5 lee 0.65 side | 1.2 | 1.95 | 0.2 and 0 | 7,310 | 191 | 3.17 | 5.19 |
| China | GD 1.0 | 0.8 | 0.5 | 1.0 (GD 1.73) | 2.0 reduction (0.87) |  | 3,148 | 108 | 1.42 | 2.84 |
| Hong Kong | 1.0 | 1.045based on drag coefficient | n/a | 1.0 | 1.4 | 0.3 and 0 | 4,986 | 127 | 1.62 | 2.27 |
| India |  | 0.7 | 0.4 lee 0.5 side |  | 1.2 | 0.3 and 0.2 | 5,442 | 141 | ??1.55 | 2.67 |
| Indonesia |  |  |  |  |  |  | 6,447 | 181 | 1.93 | 3.14 |
| Japan | GD 1.92 | (0.17Z2) 0.732 | 0.5 | 2.54 | 3 | 0.5 and 0 | 5,312 | 135 | 2.24 | 2.65 |
| Korea | GD 2.2 | 0.8 | 0.5 | 1.92 | 3.2 | 0.52 and 0 | 6,713 | 187 | 1.63 | 2.72 |
| Malaysia |  |  |  |  |  |  |  |  |  |  |
| Philippines | Cdyn 0.8 | 0.8 | 0.5 | 0.8 | 1.6 | 0.18 | 4,787 | 118 | 1.89 | 2.65 |
| Singapore |  | 0.8 | 0.5 | 1.0 | 1.95 | 0.2 and 0 | 8,804 | 219 | 3.03 | 4.93 |
| Taiwan | GD 1.71 | 0.8 | 0.5 | (GCp only)1.7 parapet 1.8 | (GCponly)3.5 parapet 1.1 | GCpi 0.375 | 3,366 | 90 | 1.39 | 2.68 |
| Thailand | 2.09 | 0.8 | 0.5 | Gc=2.5 |  | 0.3 and 0 | 3,171 | 82 | 1.94 | 1.50 |
| Vietnam | n/a | 0,8 | 0.6 lee 0.6 side | n/a | n/a | n/a | 6,676 | 181 | 3.09 | -2.32 |